ZOOM P2 Description
(for screen reader users)

Place this product on a table in front of you so that the side with the three round knobs and multiple buttons faces up. Orient it so that the smaller knob in the middle of the three is nearest you.

We will explain the parts of the P2 in the following order: the top, which has the three round knobs, the front, the left side, the right side, the bottom and the back.

Top with three round knobs

Parts are in the following order from the far end to the near end.

- There are three buttons lined up in a horizontal row.

The round button at the left end is the POWER button. Press and hold this to turn the power on or off. This lights green when the power is on.

The large rectangular button in the middle is the RECORD button that starts and stops recording. This lights red when recording.

The round button at the right end is the PLAY button. Press this to start, pause or resume playback of recorded files. Press and hold this to stop playback. Press this when recording to pause or resume recording. This lights green when playing recorded data.

- Below these three buttons is a horizontal row of five narrow buttons. From the left, these are as follows.

The button on the far left is the MUTE 1 button. This mutes input from MIC 1. This lights red when muted.

The second button from the left is the TONE button. This enables the TONE function. This lights green when the function is on. The TONE function adjusts audio from USB mics so it sounds clearer and suitable for podcasts.

The button in the middle is the AI NOISE REDUCTION button. This enables the AI NOISE REDUCTION function. This lights green when the function is on. The AI NOISE REDUCTION function analyzes the surrounding noise and suppresses its input through the USB mics.

The second button from the right is the COMP button. This enables the compressor function. This lights green when the function is on. The compressor function increases the sound pressure while avoiding clipping.

The button on the far right is the MUTE 2 button. This mutes input from MIC 2. This lights red when muted.

- Below these, the smooth rectangular area shows level indicators. The left side shows the level of MIC 1 and the right side shows the level of MIC 2 with six steps each.

- Below this area, to the left and right are 2 large round gain knobs for MIC 1 and MIC 2. The left knob adjusts the input level for MIC 1 and the right knob adjusts the input level for MIC 2. They stop turning at their minimum and maximum levels.

- Just below these in the middle is the HEADPHONE VOLUME knob. It stops turning at its minimum and maximum levels.

Front

There are two headphone jacks aligned horizontally. These are stereo mini jacks. They both output the same sound.

Left side

Near the middle is the rectangular input port for MIC 1. This is a USB Type-A port.

Right side

Parts are in the following order from the far end to the near end.

Near the middle is the rectangular input port for MIC 2. This is a USB Type-A port.

Closer to the front is the microSD card slot cover. A fingernail can be used with the notch at the top to open the cover from top to bottom. When inserting a microSD card, please orient the edge with the indentations toward the USB port so that the side with the terminals is facing down. Insert it until it clicks into place. Press the card in again to eject it.

Bottom

Please turn the unit over.

- Rubber feet are placed vertically along the left and right at the far end.

Between them are two labels, one above the other. The upper label shows a 2D code. Scanning the 2D code on the left side of this label with a smartphone, for example, allows access to a ZOOM web page with information about the P2.

The lower label shows procedures for using various settings and operations.

- Near the center, slightly closer to the front edge, is a latch for opening the battery compartment cover. Lift this latch up while pushing it to open the cover. This product uses 4 AA batteries. When installing batteries, put them in with their negative ends facing the ends with springs. When attaching the battery cover, insert the two small tabs on it in the indentations in the far end of the battery compartment. Then press the near end down until it clicks into place.

- Below and to the left of the battery cover latch is a rectangular label. To the right of the bottom of this is another small narrow label.

- At the near edge is a wide rubber foot.

Turn it over so the top is facing up and place it on the table in front of you again.

Back

From the left, the parts are as follows.

- First is the USB power port. This is a Type-C port. DC 5-volt USB AC adapters and 5-volt portable batteries can be connected. The small holes on the back are for screws.

- To the right of this, closer to the middle, is the data USB port. This is a Type-C port. When connected to a computer, smartphone or tablet, data can be transmitted and power can be supplied to the P2. The small holes on the back are for screws.

- The shape engraved below the screw holes is the ZOOM logo.

This completes this explanation of the P2 parts.

© 2025 ZOOM CORPORATION  Z2I-5795-01